Valles Caldera Line #1

In 2019 SAGE was granted a five-year permit to conduct geophysical surveys in the Valles Caldera, Sandoval Country, NM.  Valles Caldera Line #1 was the first of five seismic lines planned to be acquired in the caldera.  Line #1 was acquired in the northwest quadrant of the caldera on road VC08.  The first source point of the line (100) was located on VC08 approximately 1km west of the intersection of VC08 and VC10.  The last source point on the line (301) was located on VC08 approximately 1km west of the intersection of VC08 and VC09.

Acquisition

Source

Type					Vibroseis

Make

Ground Force

Sweep Frequencies			4-80Hz

Sweep Length				12000 ms

Start Taper				500 ms

End Taper				500 ms

Taper Type				Cos Squared

Number of Sweeps / Vibe Point	3

Number of Vibes / Source Pattern	1

Receivers

Type					3 Component

Elements				SM-7/U-B 10Hz

Number of geophones per group	1

Spread

Number of channels			360

Number of stations			120

Pattern 				Shot through fixed spread.

Source Interval			20m, on the half-station.

Receiver Interval			10m

Recording

Type					Aries XP

Data Format				SEGY Revision 0

Sample Format			IEEE Floating Point. Little Endian

Sample Interval			2 ms

Samples per Trace			2501

Record Length				5000 ms

Traces per record			363

Auxiliaries per record			3 (Channels 361, 362, 363)

Low Cut Filter				3 Hz

High Cut Filter				205 Hz

Post-stack migration of Valles Calder Line #1.   Display is from West to East.

Archive

Raw SEGY field files

00000001.SGY

00000002.SGY

00000003.SGY

00000004.SGY

.

.

.

00000179.SGY

Shell Processing Support (SPS) data

2019.sps	- 	Source Point Record

2019.rps	- 	Receiver Point Record

2019.xps	- 	Relational Record

SEGY file with source and receiver geometry in trace headers

Valle Grande #1 - geometry.sgy